Name: SWETHA S J

Dept/Sec: CCE –A

Roll no – SIT22CO051

Regno - 412422118051

Assignment - 1

1. Explain the concepts of 5G spectrum access modes in detail.

2. Explain in detail about the basics about RAN architecture and high-level requirements for 5G architecture ?

3. Evaluate the Sparse Code Multiple Access (SCMA) technique by comparing it with other non-orthogonal multiple access (NOMA) schemes.

Answers

# 5G Spectrum Access Modes

# 5G Spectrum Access Modes

- Current radio regulations provide four frequency band classifications: Exclusive bands, Shared bands, License-free bands, and Receive-only bands.

- Two service categories, Primary service and Secondary service are defined on the basis of ITU-R allocations in the regions.

- Multiple frequency bands, subject to different regulatory regimes including various forms of shared spectrum concepts as illustrated in Figure , are expected to be available for mobile communication systems.

- Thus, 5G system design requires a high degree of flexibility to be capable of operating under different regulatory models and usage scenarios.

- In general the use of radio spectrum can be authorized in two ways: Individual

- Authorization (Licensed) and General Authorization (License Exempt or Unlicensed).  Authorization modes recognized as relevant for wireless communications are Primary user mode, LSA mode and Unlicensed mode.

- Five basic spectrum usage scenarios are identified in Figure for these authorization modes:  dedicated licensed spectrum, limited spectrum pool, mutual renting,  vertical sharing, and unlicensed horizontal sharing.

- The figure depicts the relationship between parts of the domains that are either necessary or supplementary, using solid and dotted lines respectively.

- More complex spectrum usage scenarios can be supported by combinations of the basic scenarios and authorization modes

## Introduction

- Spectrum refers to radio frequency bands used for wireless communication.

- 5G requires flexible and efficient spectrum usage.

- It supports high data rate, low latency, and massive connectivity.

- Traditional licensed spectrum alone is not sufficient for 5G.

## Need for Spectrum Access Modes

- Rapid increase in mobile data traffic.

- Large number of connected devices.

- Limited availability of licensed spectrum.

- Requirement for private and enterprise networks.

- Need for efficient spectrum utilization.

## Types of 5G Spectrum Access Modes

- Licensed spectrum access.

- Unlicensed spectrum access.

- Shared spectrum access.

- Dynamic spectrum access.

- Licensed Spectrum Access

- Spectrum is exclusively allocated to a network operator.

- License is issued by government regulatory authorities.

- Provides guaranteed quality of service.

- Offers high reliability and security.

- Experiences very low interference.

- Licensing cost is very high.

- Mainly used for public 5G networks.

- Suitable for mission-critical applications.

## Unlicensed Spectrum Access

- Spectrum can be used without exclusive licensing.

- Multiple technologies share the same frequency band.

- Commonly operates in ISM bands.

- Uses Listen Before Talk mechanism to avoid interference.

- Does not guarantee quality of service.

- Has higher interference compared to licensed spectrum.

- Deployment cost is low.

- Mainly used for indoor and enterprise networks.

## Shared Spectrum Access

- Spectrum is shared among multiple users based on priority rules.

- Access is controlled through predefined sharing mechanisms.

- CBRS band is a common example of shared spectrum.

- Uses tier-based access structure.

- Managed by Spectrum Access System (SAS).

- Improves spectrum efficiency.

- Cost is lower than licensed spectrum.

- Used for private and industrial 5G networks.

## Dynamic Spectrum Access

- Spectrum is allocated dynamically based on availability.

- Users access spectrum only when it is unused.

- Uses real-time spectrum sensing techniques.

- Based on cognitive radio technology.

- Maximizes spectrum utilization.

- Reduces spectrum wastage.

- Implementation is complex.

- Suitable for dense network environments.

## Advantages of Multiple Spectrum Access Modes

- Efficient utilization of available spectrum.

- Support for diverse 5G applications.

- Reduced dependency on licensed spectrum.

- Enables private and enterprise deployments.

- Improves overall network flexibility.

- Basics of RAN Architecture and High-Level Requirements of 5G Architecture

1. INTRODUCTION

- Network Architecture integrates technical components into a unified system

- Ensures interoperability between different manufacturers.

- Logical architecture consists of Network Elements (NEs), interfaces, and protocols.

- Standardized interfaces define procedures, messages, and behavior.

- The design of network architectures aims firstly at integrating technical components into an overall system and making them properly interoperable.

- In this context, it is very important to define a common understanding on how components designed by different manufacturers are capable of communicating so that they can execute the needed functionalities.

- So far in standardization, this common understanding is achieved by the specification of a logical architecture consisting of logical Network Elements (NEs), interfaces and related protocols.

- Standardized interfaces allow for communication between NEs with aid of protocols including procedures, message formats, triggers and behavior of the logical network elements.

2. BASICS OF RAN ARCHITECTURE
 • RAN connects User Equipment (UE) to the Core Network.
 • Responsible for radio resource management and mobility.
 • In 5G it is called NG-RAN.

• RAN connects User Equipment (UE) to the Core Network.

• It is responsible for radio resource management, mobility management, and data transmission.

• In 5G, RAN is referred to as NG-RAN (Next Generation RAN).

3. KEY COMPONENTS OF 5G RAN
• User Equipment (UE): Mobile device used by the end user.

• gNodeB (gNB): 5G base station that communicates with UE.

• Central Unit (CU): Handles higher-layer protocols such as PDCP and RRC.

• Distributed Unit (DU): Handles real-time MAC and PHY functions.

• Radio Unit (RU): Performs RF processing and transmission..

4. FUNCTIONAL ARCHITECTURE
 • Network Functions (NFs) perform operations.
 • Each NF is mapped to a Network Element.
 • Different NFs have different latency and bandwidth needs.

5. PHYSICAL ARCHITECTURE
 • Physical Architecture maps NEs and NFs to physical nodes.

• Placement impacts network performance and network economics.

• Centralized Placement supports statistical multiplexing.

• Distributed Placement reduces latency near the air interface.

6. ROLE OF SDN AND NFV
 • Network Function Virtualization (NFV) virtualizes network functions.

• Software Defined Networking (SDN) separates control and data planes.

• Enables flexible placement and dynamic orchestration of NFs.

• 5G focuses more on Network Functions than Network Elements

7. HIGH LEVEL REQUIREMENTS OF 5G
 • Enhanced Mobile Broadband (eMBB): High data rates and capacity.

• Ultra-Reliable Low Latency Communication (URLLC): Low latency and high reliability.

• Massive Machine Type Communication (mMTC): Supports IoT devices.

• Low Latency: End-to-end latency below 1 ms.

• High Reliability: 99.999% availability.

• Scalability: Supports massive number of users and devices.

• Flexibility: Dynamic network slicing and service deployment.

• Energy Efficiency: Reduced power consumption.

- Evaluation of Sparse Code Multiple Access (SCMA) and Comparison with Other NOMA Schemes

## 1. Introduction to NOMA

With the evolution of 5G and beyond (B5G/6G) networks, traditional Orthogonal Multiple Access (OMA) techniques such as TDMA, FDMA, CDMA, and OFDMA are insufficient to meet requirements like:

- Massive connectivity

- Low latency

- High spectral efficiency

- Support for IoT and mMTC

To overcome these limitations, Non-Orthogonal Multiple Access (NOMA) schemes were introduced, allowing multiple users to share the same time-frequency resources.

Sparse Code Multiple Access (SCMA) is one of the most promising code-domain NOMA techniques.

## 2. What is Sparse Code Multiple Access (SCMA)?

SCMA is a non-orthogonal, code-domain multiple access technique where:

- User data is directly mapped to multi-dimensional sparse codewords

- Multiple users share the same time–frequency resources

- Overloading is supported (users > resources)

### Key Idea

Each user is assigned a unique sparse codebook, and only a few resource elements (REs) are used by each codeword → reducing interference.

## 3. SCMA Transmitter and Receiver Operation

### 3.1 Transmitter Side

- Input bits are taken from each user

- Bits are directly mapped to sparse codewords

- Codewords are spread across multiple subcarriers

- Signals from multiple users are superimposed

### 3.2 Receiver Side

- Uses Message Passing Algorithm (MPA)

- Exploits sparsity of codebooks

- Performs near-optimal multi-user detection with low complexity

## 4. Sparse Nature of SCMA

- Each codeword has many zero elements

- Only a few subcarriers are active per user

- Leads to:

  - Reduced inter-user interference

  - Lower detection complexity

  - Efficient resource utilization

## 5. Advantages of SCMA

### 5.1 High Spectral Efficiency

- Supports overloading factor > 100%

- More users served per resource block

### 5.2 Massive Connectivity

- Ideal for IoT and mMTC

- Large number of users can be connected simultaneously

### 5.3 Low Latency

- Grant-free access possible

- Suitable for URLLC applications

### 5.4 Improved Detection Performance

- MPA provides near-maximum likelihood performance

- Better BER compared to other NOMA schemes

### 5.5 Robust to Noise and Interference

- Multi-dimensional modulation improves reliability

## 6. Limitations of SCMA

- Complex codebook design

- Higher receiver complexity than OMA

- Performance depends heavily on optimized MPA implementation

## 8. Applications of SCMA

- 5G New Radio (NR)

- Massive Machine Type Communication (mMTC)

- Internet of Things (IoT)

- Ultra-Reliable Low Latency Communication (URLLC)

- Smart cities and Industry 4.0

Sparse Code Multiple Access (SCMA) is a powerful code-domain NOMA technique that enables:

- Massive connectivity

- High spectral efficiency

- Low latency communication

Compared to other NOMA schemes such as PD-NOMA, LDS-CDMA, and MUSA, SCMA offers better performance, reduced interference, and improved reliability, making it a strong candidate for 5G and future wireless systems.